## BUILDING PROTOTYPE SPECIFICATIONS

## Choosing Targeted Requirements

Note: The requirements we are keeping are shaded in green, and the ones we are discarding are shaded in red. Justification for keeping/discarding requirements for our prototype is given in the rightmost column. The goal is to produce a prototype of the entire system, with a focus on demonstrating the system's automation of water flow. Why are we prototyping automation of water flow? - Our design can roughly be split into two components: water heating via a parabolic trough with single-axis solar tracking, and water movement and storage via temperature, water level sensors, valves, and storage containers. - These components have already been shown to work individually, in multiple different designs/applications. For example: - Parabolic trough heating is successful for water heating (up to even >400ºC) in both residential and commercial/industrial spaces. It’s also been successful at cooking food (ex. Solar BBQ). Parabolic trough solar tracking has been done as well. - Automated water movement through a piped system, including automated temperature-based valves, is done in HVAC systems, plumbing systems, and irrigation. - What hasn’t been done is the specific combination of a parabolic trough and automated water movement to the specifications we have listed in our requirements (i.e. heat water at a specified, permanently set temperature and time, dispense at room-temp water). - So, we want to prototype the entire system to test this combination but it’s difficult to make and test a working parabolic trough (as described in the justification for the first functional requirement in the table below). So, we will focus on the functionality of automated water movement. We’ll still make a trough with solar-tracking movement, but we won’t be as concerned about whether it can heat water.

## Primary Functional Requirements

| Objective | Metric(s) | Criteria | Constraint(s) | Justification for Keeping/Discarding Requirement |
| --- | --- | --- | --- | --- |
| Should heat water completely through solar energy | True/False | --- | Must be true (the heating must be powered by solar energy, but there is flexibility for other electronic components) | It’ll be difficult to prototype a trough that heats the water any significant amount (ex. more than a degree ºC or two). This is because existing parabolic troughs have a lot of characteristics that are hard to reproduce, ex. - Heating fluid (ex. antifreeze) pumped continuously – we can’t get access to this heating fluid - Large size (1-15+ m) - Curved reflective mirrors We also don’t want to have to rely on there being warm, sunny days in the next few weeks to test our prototype. Multiple reference designs exist that show that parabolic troughs can heat water to our specifications (see Notion reference design research), so we wouldn’t really be “proving” anything new by prototyping it. Instead, we can have an externally heated supply of water on hand to test the rest of the system. |
| Must keep water at target temperature in heating tank for a satisfactory time interval | Time (minutes) | Longer is better up to more than 2 minutes | Must keep water at target temperature in heating tank for at least 2 minutes | This is a key step in showing how water flows through the system, since water movement out of the heating tank is time-controlled (as well as temperature-controlled). |
| Must heat water up to a satisfactory target temperature | Temperature (Celsius) | Plateau from 70-100°C. Higher is worse after 100°C. | Must get water temperature to 70°C Should not get water temperature above 100°C | Heating water up to 70ºC isn’t necessary to show how water flows through the system as the materials will sustain the temperature. |
| Should store sanitized water | True/False | --- | Must be true | This is a key step in showing how water flows through the system. |

## 1.2	DfX and other Requirements

## Requirements are grouped under High-Level Objectives (HLOs).

| Objective | Metric(s) | Criteria | Constraints | Justification for Keeping/Discarding Requirement |
| --- | --- | --- | --- | --- |
| HLO 1: Design for Efficiency |  |  |  |  |
| Must use solar power efficiently | Thermal efficiency: ηth = Qout/Qin | Higher is better | Must be at least 45% | Even though we won’t make a functional trough, we want to make a reproduction of a trough that’s accurate, and so thermal efficiency should be considered where possible in its design. In reality, thermal efficiency is hard to calculate, and doesn’t seem worth the effort to estimate if we aren’t concerned with trough performance. Alternative assessments of thermal efficiency can be made, however (observing any unwanted heat losses, etc.). |
| Heating tank should be thick enough to be efficiently insulated | R value (a measure of thermal resistance) (K⋅m^2/W) | Higher is better | Depends on the materials and the dimensions (see standards) | Not necessary to show how water flows through the system. |
| Solar collector must be angled toward the Sun efficiently | Solar collector tilt angle in degrees | --- | If fixed, it must be equal to the local latitude. If not fixed, it should follow the direction of maximal sunlight throughout the day. | If we have time, it would be cool to prototype the rotation of the trough! Seems relatively easy to do as well, given MyFab materials like motors. |
| HLO 2: Design for Affordability |  |  |  |  |
| Should be low material cost | Cost (CAD) | Lower is better | --- | We don’t have a big budget anyway :) |
| Should have low maintenance costs | Cost (CAD) | Lower is better | --- | We aren’t concerned with the long-term maintenance of the prototype. |
| HLO 3: Design for Safety |  |  |  |  |
| Water is dispensed at a safe temperature | Temperature (Celsius) | Downward parabola, zeroes at 10 and 49°C | Should be below 49°C Should be higher than 10°C | This is the outcome of a successful water flow mechanism. |
| Any component that comes into contact with water must not transfer harmful chemicals into the water | True/False | --- | Must be true | We have water-safe materials available to us through MyFab. |
| Water heating components must be made of materials able to withstand high temperatures | Creep limit | Higher is better | Must not creep up to 100 degrees Celsius | MyFab doesn’t have temperature-resistant versions of everything we need (pumps, sensors etc.), and using very hot water isn’t necessary for our prototype anyways. |
| Must seal water storage containers when not in use | Yes/No | --- | Must be yes | It’ll be easier to demo our prototype by leaving containers open, since we can more clearly show water flow, temperature measurement, etc. |
| Electrical components should not overheat under normal operating conditions | Temperature in Celsius | --- | Refer to relevant datasheets (depends on sensors used) | It’ll be important to incorporate our electrical components in a way that doesn’t expose them to the hottest parts of the system – this is an important design consideration even if we aren’t using very hot water, since electrical component placement would transfer to the design concept as well. |
| HLO 4: Design for Reliability |  |  |  |  |
| Should be durable | Time until failure (years) | Higher is better | Should last for at least 20 years | Our prototype doesn’t need to be so durable. It isn’t necessary to show system functionality. |
| Must operate for necessary number of days | Number of days of the school year (days) | Higher is better | Must work for the days within individual school terms of a cumulative total of 210 days per year | We can design our prototype such that nothing, in theory, stops it from working for this long. But, since actually testing that the prototype works for so long is impossible, this isn’t an important consideration. |
| HLO 5: Design for Usability |  |  |  |  |
| Must disinfect enough water for daily school use | Water disinfection capacity (L/day) | Higher is better up to 10kL. (Plateaus after 10kL) | Should be at least 5kL | Such a large-scale implementation is unnecessary. |
| Any components that are meant to be moved frequently by users should be lightweight | Weight of component (kg) | Lower is better | Should be less than 16 kg | No components are meant to be moved frequently by users, so this passes by default. |
| Must display the amount of stored sanitized water to the user(s) | Yes/No | --- | Must be yes | Since the design’s containers will be transparent, it is unnecessary to investigate this functionality |
| Should be easy to maintain | # of cleaning instances | Lower is better | --- | We aren’t concerned with the long-term usability of the prototype. |

## Targeted Requirements Flow-Down

## 2.1. Parabolic Trough

## Note: The requirements removed for this subsystem are: ## Should store sanitized water (since this isn’t a part of this subsystem) ## Must use solar power efficiently (since this is captured in “should heat water” requirement, as explained in table below) ## Water is dispensed at a safe temperature (since this isn’t a part of this subsystem)

| Objective | Metric(s) | Criteria | Constraints | Subsystem-Specific Justification |
| --- | --- | --- | --- | --- |
| Primary Functional Requirements |  |  |  |  |
| Should heat water | Temperature difference between outlet and inlet of trough pipe (∆T=Toutlet - Tinlet) | Greater is better | None | It would be good if the trough could actually heat water, and it makes sense to design the trough so that it can theoretically heat water as much as possible. But given the difficulties described in Section 1 justification for getting rid of heating requirements, this isn’t a huge priority for us, so we use “should” instead of “must” and no constraints are set. Note: The “must use solar power efficiently” requirement is absorbed into this requirement, since higher thermal efficiency translates directly into higher temperature difference if we’re only considering the trough (i.e.. any wasteful heat losses in the trough itself would be captured in a lower temperature difference). |
| HLO 1: Design for Efficiency |  |  |  |  |
| Solar collector must be angled toward the Sun efficiently | Solar collector tilt angle in degrees | --- | It should follow the direction of maximal sunlight throughout the day. | Solar tracking is part of the trough subsystem. |
| HLO 2: Design for Affordability |  |  |  |  |
| Should be low material cost | Cost (CAD) | Lower is better | --- | --- |
| HLO 3: Design for Safety |  |  |  |  |
| Any component that comes into contact with water must not transfer harmful chemicals into the water | True/False | --- | Must be true | --- |
| Electrical components should not overheat under normal operating conditions | Temperature in Celsius | --- | Refer to relevant datasheets (depends on sensors used) | --- |

## 2.1.2. Water Movement and Storage

## Note: The requirements removed for this subsystem are: ## Solar collector must be angled toward the Sun efficiently (since this isn’t a part of this subsystem) ## Must use solar power efficiently (since this isn’t a part of this subsystem, especially because we don’t care about the heating tank being insulated)

| Objective | Metric(s) | Criteria | Constraints | Subsystem-Specific Justification |
| --- | --- | --- | --- | --- |
| Primary Functional Requirements |  |  |  |  |
| Must keep water at target temperature in heating tank for a satisfactory time interval | Time (minutes) | Longer is better up to more than 2 minutes | Must keep water at target temperature in heating tank for at least 2 minutes | Sensors must successfully detect the targeted temperature. |
| Should store water | True/False | --- | Must be true | Shows that water can be delivered to the users; ensures the subsystems are successfully integrated |
| HLO 2: Design for Affordability |  |  |  |  |
| Should be low material cost | Cost (CAD) | Lower is better | --- | Has to meet budget requirement of Phase 3 |
| HLO 3: Design for Safety |  |  |  |  |
| Water is dispensed at a safe temperature | Temperature (Celsius) | Downward parabola, zeroes at 10 and 49°C | Should be below 49°C Should be higher than 10°C | Sensors must successfully detect the targeted temperature. Dispense water only when the detected temperature is lower than a certain level. |
| Any component that comes into contact with water must not transfer harmful chemicals into the water | True/False | --- | Must be true | Use water-safe materials from MyFab. |
| Electrical components should not overheat under normal operating conditions | Temperature in Celsius | --- | Refer to relevant datasheets (depends on sensors used) | --- |

## Translating Flowed-Down Requirements to the Prototype System

## 3.1. Parabolic Trough

## Functional Goals:

## 1. Accurate visualization of one parabolic trough that would be implemented in this scaled design concept with its major components

## 2. Link the trough to the rest of the system (The link is more important than functional integration --> Pipe runs from trough to boiler tank with nonfunctional heating fluid)

## 3. Automatically rotate based on sun tracking (Pre-fed algorithm is not important here as it can be bought and is well tested, Trough frame should rotate 270 degrees back and forth to demonstrate autonomous rotation)

## Overall takeaways:

## What aspects of the Design Concept will be embodied, and how? Which will not?

## single-axis solar tracking will be implemented, probably via stepper motors (precise)

## heating will probably NOT be implemented (at least, that’s not the goal)

## overall “look” of parabolic trough will be implemented, incl. dimensions, equivalent materials, scaled down

## How does this affect what you will learn from prototyping?

## will learn how to scale down a design while preserving/including important properties (relative dimensions, weight, material properties, durability when rotated, etc.)

## will learn how to integrate simple constructed model + actuators, how to control actuators with microcontrollers + write software for actuators

## will NOT learn how to design out heat losses, test heat losses, test heating effectiveness, etc.

## 3.2. Valves

## First valve between boiler and hot tank

## Second valve between hot and cold tank

## Functional Goals:

## 1. Implement valve regularity by demonstrating opening/closing as required through full integration

## 2. Keep water in boiler tank for a sufficient time interval

## 3. Valve should completely close tube to prevent water flow as required

## DfX:

## Affordability: Low material cost (should fall within the budget, with the other subsystems)

## Safety:

## Water is dispensed at a safe temperature (second valve)

## Watertight so that contaminated water does not flow through when it shouldn’t (first valve)

## Made of water safe materials that does not introduce harmful chemicals (both valves)

## Electrical components should not overheat under normal operating conditions (both valves)

## Inputs:

## Temperature data from feeder tank + water supply

## Water level data from receiving tank

## Outputs:

## Power from motor (controlled by microcontroller) --> Prevent waterflow, or allow waterflow (based on inputs)

## Overall takeaways:

## What aspects of the Design Concept will be embodied, and how? Which will not?

## Automated water flow based on sensors will be embodied (this is the main goal of the overall prototype) by the processing of sensor data and motor outputs based on sensors (seen in the valve)

## Functionality of the valve will be embodied, since it is automated (like in the design concept) and should effectively stop water flow when closed

## Will be gravity fed, like the true design concept

## Will NOT embody the true scale of the design concept, control the same temperature and duration of water sanitization process as the design concept

## How does this affect what you will learn from prototyping?

## This will help us gain a more nuanced understanding of how well we might be able to control the water flow

## This prototype will help us understand signal processing from thermistors and water level sensors and whether they will be sufficient to automate the water flow

## This will NOT give use information on whether, in practice, the materials and sensors will be able to handle high temperatures of water, if the true scale of the design concept will be feasible (it will be smaller in scale)

## 3.3. Boiler, Hot, and Cold Tanks

## Functional goals:

## Provide visualization of water-cooling process

## How water flows from one to another

## Integration with other subsystem such as valves and trough

## Ie. Location of the valves that connect the tanks

## Provide visualization of connection between three separate tanks

## Methods of water flow

## Tanks’ arrangement

## Inputs:

## Water

## Outputs:

## Cooled-down water

## Expect a small decrease in temperature considering limitations due to the scope/size of the prototype

## Overall takeaways:

## What aspects of the Design Concept will be embodied, and how? Which will not?

## The process of water cooling via flowing through separate tanks will be implemented; using 3D-printed tanks and valves that control the water flow

## The decrease in water temperature is expected but temperature threshold will be adjusted; temperature measurement done using thermistor and cooling using natural convection

## The arrangement of the tanks will be implemented considering the feeding/water flow mechanism

## Mechanism of storing water, from each heating cycle, separately will be implemented using valves and leak free tanks

## How does this affect what you will learn from prototyping?

## Will learn how to integrate with other subsystems in smaller setting

## Will learn what aspects of user interface to consider or prioritize and how to improve the design to optimize the user comfort

## I.e. The location of the cold tank will be significant to ensure that users can collect from it. The location of the cold tank is more important than the locations of the others

## Will gain more solid 3D-printing skills such as parameters (attention to detail), as it requires integration with other subsystems, and material selection, to ensure the durability and security of stored water

## 3.4 Water Movement and Storage

## Functional goals:

## Must detect water level and temperature successfully.

## Must send signals to the other subsystems (i.e., valves) to dispense water only when the water level and temperature reaches a certain level of threshold.

## Must not send false-positive or true-negative signals to other subsystems.

## Inputs:

## Water

## Outputs:

## Measured temperature data

## Measured water level data

## Send signals to the subsystems

## Overall takeaways:

## What aspects of the Design Concept will be embodied, and how? Which will not?

## The automated water flow using the sensors will be implemented

## Will detect the water level and temperature (like our Design Concept)

## Measured data will be sent to the valves to control the water flow

## How does this affect what you will learn from prototyping?

## Will learn how to calibrate data

## Will learn how to integrate with other subsystems

## Will learn how to integrate sensors

## Will gain more experience with CircuitPython codes

## Creating a Specification for your Prototype

## Please note that specifications that are to be actively tested are further explored under subsystem testing protocols.

## 4.1. Parabolic Trough (rough)

## Water must be able to flow through the trough pipe.

## The trough must rotate in an effective manner (timing, regularity, amount of rotation) for solar tracking.

## The trough pipe must be connected to the rest of the system (heating tank, etc.).

## The trough must be a faithful representation of a typical functioning parabolic trough (ie. major components represented, accurate dimensions).

## 4.1. Parabolic Trough (final)

## Mechanical:

- The trough must be securely attached (no wobbling) to a stepper motor shaft, and its rotation (˚ degrees) must match that of the shaft.

- The trough pipe must run through the trough in a straight line through two holes on either flat end of the trough. The trough pipe must form a loop, and be wrapped around the heating tank --> this models the design concept, where heating solution will be pumped continuously through the trough pipe, transferring heat to the tank in the process.

- The trough must be a faithful visual representation of a typical functioning parabolic trough with scaled down/substituted major components. This consists of:

  - The trough must consist of a parabolic plane with two closed ends, each with one tubing hole.

  - The trough shape must follow a parabola of the form x^2=4fy, where f is the focal length of the trough (ie. where trough holes are located).

  - The inside of the trough’s parabolic plane must be covered in a shiny and/or reflective material.

- The trough’s rotation through all 3 modes (described in Software section) must not interfere with layout or function of rest of system.

## Electrical:

- Must provide sufficient power to move the motor that in turn rotates the weight of the trough frame

- Electrical components should not pose safety hazards (contact with water mainly)

- Wires must be cleanly organized and colour-coded (ex. red for power, black for ground).

- Wires must be securely attached (no detaching) to pins

Software:

- Must implement cyclic formation controlled by a button that cycles between the trough’s shut down mode, autonomous rotation mode, and stationary mode.

- Must light up LED to indicate autonomous rotation mode --> since the true design concept trough will rotate very slowly, this will provide as an indication that this mode is active.

- Must be able to detect baseline of 0-degree trough rotation through a limit switch signal.

- Must be able to recalibrate and re-assume correct motion (according to current mode) upon limit switch activation only -> edge cases that don’t activate the limit switch are un-detectable

- Must be able to assume rotation-downward mode upon HIGH signal from water level sensor Nano, and must be able to resume previous state upon LOW signal.

## 4.2 Valves (also still very high-level for now)

## Mechanical:

## Water must flow through valve from one tank to the next when the valve is open, and circulation must be fully stopped when the valve is closed

## Must be gravity fed (will require a downwards incline from tank to tank)

## Electrical:

## Must provide sufficient power to move the motor and close the valve completely

## Must automatically detect open and close the valve without external help

## Electrical components should not pose any safety hazards (overheating, contact with water, using the proper resistors)

## Software:

## Valve must open after detecting a specified temperature for a specified length of time in the feeder tank, detected by thermistor

## Valve should only open if the receiving tank has sufficient space to take in water, detected by the water level sensor, includes proper and accurate signal filtering of sensors

## 4.2. Valves (final)

## Actuators & mechanisms:

## Water must flow through valve from from one tank to the next at a reasonable flow rate (at most 120 seconds to fill each 600 mL tank).

## Valve must demonstrate a visible restriction in flow rate, ideally cutting off the water flow completely.

## Motor must have sufficient torque and power to move and close the valve.

## Valve must be controlled continuously, without manual adjustments and manual indication of whether it is fully closed (automatic detection of closed of open valve)

## Must be gravity-fed

## Must be leak-free and remain uncontaminated

## Able to be well integrated into the rest of the system (not too big)

## Electrical components should not pose any safety hazards (overheating, contact with water, using the proper resistors).

## Motor and valve must be firmly held in place in the overall prototype structure.

Software & Automation:

- Valve must open after detecting a specified temperature for a specified length of time in the feeder tank, input --> thermistor

- Valve must only open if the receiving tank has sufficient space to take in water, input --> water level sensor

## 4.3 Boiler, Hot, and Cold Tanks

## Hot & Cold Tanks:

## Water flowing into cold tank must be sufficiently cooled down

## Water flow must be operated via valves only

## Must not leak or overflow

## Must display and measure water level in the cold container

## Boiler Tanks:

## Must be able to store hot water for 2 minutes

## 4.3. Boiler, Hot and Cold Tanks (final)

## Water flowing into the cold tank must be sufficiently cooled down (preferrably room temperature)

- Containers must not leak, i.e. water flow must only be operated via valves and tubing

- Must measure water level in storage

- Must be able to store hot (specified temperature) water for specified time interval

## 4.4 Water Movement and Storage

## Water must be able to flow through the system using only gravity

## Should hold less than or equal to a specified amount of water in each tank

## Structure to support system should hold the weight of the specified system

## 4.4 Water Movement and Storage

## Water must be able to flow through the system using only gravity

## Should hold less than or equal to a specified amount of water in each tank

## Structure to support system should hold the weight of the specified system and provide appropriate height for the gravity fed mechanism