Parabolic Trough Prototype Testing Protocol and Results

1. Specification 1: Autonomous Rotation

The prototyped trough should rotate on its horizontal axis using a stepper motor based on software specifications to verify rotation automation capabilities.

1.0. Normal Operation

This test is meant to simulate expected operation of the trough. All 3 stages (ie. modes of operation) are shown.

Testing Protocol

Note that # of degrees is defined as increasing clockwise from zero-degree position (trough facing down).

- Establish baseline by initially positioning the trough at 0 degrees facing down, specified by the limit switch signal.

- Plug Pico into computer. Load the CircuitPython program to power system on. Observe by referring to Table 1, A1.

- Press the button in the circuit.

- Observe result and compare against expected result. Refer to Table 1, A2.

- Press the button in the circuit.

- Observe result and compare against expected result. Refer to Table 1, A3.

- Power system off by disconnecting Pico from computer. Observe by referring to Table 1, A4.

Table 1: Collected Data from Testing Protocol Specification 1

For video of this test, see video titled “Test1_0” in Test Videos folder. Please refer to video timestamps in rightmost column.

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Power system on | Trough remains stationary at 0 degrees | As expected [00:00-00:14] |
| A2. Button press | Trough rotates to 270 degrees, then rotates between 90 degrees and 270 degrees repeatedly | As expected [00:14-00:41] |
| A3. Button press | Trough rotates back to 0 degrees, then rotates to 90 degrees | As expected [00:41-00:50] |
| A4. Power system off | Trough becomes stationary | As expected [00:50-01:09] |

1.1. Edge-Case 1: Unexpected Manual Trough Rotation

This test is meant to simulate cases where the trough rotates, or is unable to rotate, unexpectedly (ex. because of excessive wind, collected debris in trough, motor jamming, obstacles blocking rotation path, or human intervention).

Testing Protocol

- Have trough at Stage 1. Rotate trough by hand toward/away from zero position by various amounts multiple times. Observe result and compare against expected result. Refer to Table 2, A1.

- When trough is away from zero position, block trough by hand for a few seconds, then remove hand. Observe result and compare against expected result. Refer to Table 2, A2.

- Switch to Stage 2 by button-press. Rotate trough CCW such that the trough’s path overlaps with the zero position. Observe result and compare against expected result. Refer to Table 2, A3.

- Rotate trough multiple times (both CCW and CW) such that the trough’s path does not overlap with the zero position. Observe result and compare against expected result. Refer to Table 2, A4.

- Re-zero trough to allow normal 90-270˚ rotation to occur. Block trough by hand for a few seconds, then remove hand. Observe result and compare against expected result. Refer to Table 2, A5.

- Switch to Stage 3 by button-press. After trough reaches final upward position, rotate back to zero position. Observe result and compare against expected result. Refer to Table 2, A6.

- Re-zero trough. After trough reaches final upward position, rotate varying amounts without going back to zero position. Observe result and compare against expected result. Refer to Table 2, A7.

Table 2: Collected Data from Testing Protocol Edge-Case 1.1

See video titled “Test1_1” in Test Videos folder.

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Stage 1 random rotation | Trough rotates to 0 degrees | As expected [00:00-00:20] |
| A2. Stage 1 blocked rotation | Trough rotates to 0 degrees | As expected [00:20-00:26] |
| A3. Stage 2 significant CCW rotation | Trough resets to correct 90-270˚ rotation after zeroing | As expected [00:26-1:00] |
| A4. Stage 2 slight CCW and CW rotation | Trough rotation degree is off by the amount manually rotated | As expected [1:00-1:15] |
| A5. Stage 2 blocked rotation | Trough rotation degree is off by the amount of rotation missed | As expected [1:15-1:37] |
| A6. Stage 3 rotation back to zero | Trough rotates back upward | As expected [1:37-1:58] |
| A7. Stage 3 random CCW and CW rotation, without going back to zero | Trough degree is off by the amount rotated | As expected [1:58-2:06] |

1.2. Edge-Case 2: Button Interactions

This test is meant to cover cases of how the user might interact with the state-change button, including holding and spamming.

Testing Protocol

- Have trough at Stage 1. Hold button for 5 seconds, then release. Observe result and compare against expected result. Refer to Table 3, A1.

- Have trough at Stage 2. Press button quickly 10 times. Observe result and compare against expected result. Refer to Table 3, A2.

- Have trough at stage 3. Try a random combination of fast and slow button presses and holds, for 10 presses. Observe result and compare against expected result. Refer to Table 3, A3.

Table 3: Collected Data from Testing Protocol Edge-Case 1.2

See video titled “Test1_2” in Test Videos folder.

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Button hold | Trough moves to Stage 2 immediately upon button press. No reaction upon button release. | As expected [00:00-00:10] |
| A2. Button spam | Trough cycles through stages as button is pressed, and ends on Stage 3. | As expected [00:10-00:27] |
| A3. Random combination of button hold + spam | Trough cycles through stages as button is pressed, and ends on Stage 1. | As expected [00:27-00:44] |