Below is the calculation for the valid current-limiting resistor range of a green LED connected to a Raspberry Pi Pico.

GREEN LED (spec sheet https://www.kingbrightusa.com/images/catalog/SPEC/WP7113SGD.pdf)

- target luminous intensity relative to 20mA: 0.2

- max current: 14mA

  - fwd voltage: 2.2V

  - actual voltage: 2.62-3.3V

  - current-limiting resistance: (3.3-2.2)/0.014 = 79 ohms

- min current: 4mA

  - fwd voltage: 2V

  - actual voltage: 2.62-3.3 V

  - current-limiting resistance: (3.3-2)/0.004 = 325 ohms

- valid resistor range: 79-325 ohms

- resistor chosen: 220 ohms