Testing Protocol for Valve Actuators and Mechanism

Preliminary Tests

After printing the first design version, I made a series of preliminary tests relating to specification 1 and 2 (both in the water flow preliminary test), specification 3 (torque test) and specification 4 (valve control). These were mostly informal and simply used to inform future designs. For example, the torque test failed for the micro servo but was okay with the servo MG966. Therefore, I designed the next valve to fit this servo motor.

Final Tests

I performed a series of tests for the final valve, based on the system specifications.

## 1. Water must flow through valve from one tank to the next at a reasonable flow rate (should take at most 120 seconds to fill a 600 mL tank).

1.1 Flow rate tests

The purpose of this test is to compare the flow rates of the open valves when they are arranged in a different default position. It should help decide on a design and also confirm the viability of a design.

Testing protocol:

For the test, I filmed and timed the amount of time it took for water from the tube (attached to the appropriate attachments on my 3D printed piece) would take for both the straight and curved attachments.

Controlled variables:

- Water from the faucet was running at the same intensity (constant water pressure)

- The placement and size of the receiving cup (about 400 mL)

- The height of the tube and the length of the tube.

Independent Variable: The arrangement of the tube in the 3D printed piece (straight or curved and partially pinched by the print shape).

Dependent/Measured Variable: Time the water takes to fill up the cup.

Testing Procedure:

- Find a water source (e.g. a sink)

- Set up the valve with the tubing so that it is in the default open position (whatever that may be for this specific trial).

- Place a cup/bowl of known volume capacity below the valve.

- Turn on the water tap, place the tubing below it (with the valve under as well) and record the amount of time it takes to fill the cup/bowl.

Table 1: Collected Data from Testing Protocol Specification 1

For video of this test, see video titled “Test1_0” in Test Videos folder. Please refer to video timestamps in rightmost column.

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Using the curved valve, turn on the tap and place the tubing and valve underneath. | Water should pass through at a rate faster than 120 seconds per 600mL (as per specification). | Below expected – 111 seconds for 400 mL, which is 166.5 seconds for 600 mL [00:00-01:51] |
| A2. Using the straight valve, turn on the tap and place the tubing and valve underneath. | Water should pass through at a rate faster than 120 seconds per 600 mL (as per specification). | As expected – 57 seconds for 400 mL, which is 85.5 seconds for 600 mL [00:00-00:57] |

Therefore, changing the arrangement of the tubing could increase the flow rate significantly. Choosing the straight valve arrangement could be beneficial for testing and demos later on.

2. Valve must demonstrate a visible restriction in flow rate (ideally fully cut off water flow completely).

2.1 Pinching mechanism test: This test is a test for the mechanism behind pinching the tube to determine whether it is sufficient to close off water flow. I mainly used it to compare the performance of two potential designs. The options this was tested on are provided in the image of this test’s respective folder.

Testing protocol

- Connect flexible tubing to the bottom hole of one of the tanks.

- Place tank on an elevated surface.

- Fill the tank with water.

- Check baseline flow of the water from the tubing without any valve interference.

- Connect the tubing into the valve (without the motor), so that it is ready to be pinched with your hands.

Option 1

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Let water run. | Water should run normally. | As expected [00:00-00:20] |
| A2. Pinch valve with the mechanism. | Water flow should be restricted. | As expected, water flow was fully closed off [00:20-00:27] |
| A3. Release | Water flow should resume. | As expected [00:27-00:35]` |
| A4. Pinch again briefly. | Water flow should be restricted. | As expected [00:35-00:37] |
| A5. Release. | Water flow should resume. | As expected [00:37-00:40] |
| A6. Pinch | Wate flow should be restricted. | As expected [00:40-00:41] |
| A7. Release | Water flow should resume | As expected [00:41-00:43] |
| A8. Pinch | Water flow should be restricted. | As expected [00:43-00:49] |

Option 2

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Let water run. | Water should run normally. | As expected [00:00-00:13] |
| A2. Pinch valve with the mechanism. | Water flow should be restricted. | As expected, water flow was mostly closed off [00:13-00:17] |
| A3. Release | Water flow should resume. | As expected [00:17-00:21]` |
| A4. Pinch again briefly. | Water flow should be restricted. | As expected, water flow was mostly closed off [00:21-00:24] |
| A5. Release. | Water flow should resume. | As expected [00:24-00:28] |
| A6. Pinch | Wate flow should be restricted. | As expected, water flow was mostly closed off [00:28-00:30] |
| A7. Release | Water flow should resume | As expected [00:30-00:41] |
| A8. Pinch | Water flow should be restricted. | As expected, water flow was mostly closed off [00:41-00:44] |
| A9. Release | Water flow should resume | As expected [00:44-00:45] |
| A10. Pinch | Water flow should be restricted. | As expected, water flow was mostly closed off [00:45-00:46] |

## 3. Motor must have sufficient torque and power to move the motor and close the valve.

This was tested for along with the fourth specification as described in the following protocol.

4. Valve must be controlled continuously, without manual adjustments and manual indication of whether or not it is fully closed (automatic detection of closed or open valve).

4.1 Free rotation without external contact.

- Clamp the motor and lay level with the valve mechanism (flat on a table).

- Confirm that internal rotating component can rotate freely without contact with the table.

- Connect the circuitry and connect the code.

4.1.1 Regular open and close of valve

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Without touching the valve, close the valve (with the code). | Valve should close (180 degrees counterclockwise) | As expected [00:10-00:14] |
| A2. Wait a couple seconds | Nothing should change. | As expected [00:14-00:16] |
| A3. Open valve. | Valve should open (rotate 180 degrees clockwise) | As expected [00:16-00:20] |
| A4. Wait a few seconds | Nothing should change. | As expected [00:20-00:26] |

4.1.2 Quick open and close of valve

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Without touching the valve, close the valve (with the code). | Valve should close (180 degrees counterclockwise) | As expected [00:00-00:06] |
| A2. Open valve immediately after. | Valve should open (rotate 180 degrees clockwise) | As expected [00:06-0:18] |

4.1.3 Continuous open and close of valve (making sure the valve can endure continuous strain and rotation and other edge cases).

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Without touching the valve, close the valve (with the code). | Valve should close (180 degrees counterclockwise) | As expected [00:00-00:26] |
| A2. Wait a couple seconds | Nothing should change. | As expected [00:26-00:35] |
| A3. Open valve. | Valve should open (rotate 180 degrees clockwise) | As expected [00:35-00:38] |
| A4. Close valve | Valve should close (180 degrees counterclockwise) | As expected [00:38-00:41] |
| A5. Wait a few seconds | Nothing should change. | As expected [00:41-00:53] |
| A6. Open valve | Valve should open (rotate 180 degrees clockwise) | As expected [00:53-00:58] |
| A7. Close valve | Valve should close (180 degrees counterclockwise) | As expected [00:58-1:05] |
| A8. Twitch in motor (not at 0 or 270 degrees from initial motor angle) | Valve should remain closed. | As expected [1:05-1:09] |
| A9. Rotation back and forth between 90 and 180 degrees from initial motor angle) | Valve should remain closed. | As expected [1:09-1:13] |
| A10. Open valve | Valve should open normally (rotates back to initial position) | As expected [1:13-1:27] |

4.1.4 Edge case obstruction of rotating component

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Place valve in a way that obstructs the rotation of motor. | Will not be able to rotate. Nothing should fall apart. | As expected, motor stopped, valve lifted slightly off the table but no damage was done to the valve [00:00-00:08] |

4.1.5  Testing for extra pressure on sides and continuous rotation (both edge cases)

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Placing two fingers, one on each side of valve and push down, close valve. | Valve should close (180 degrees counterclockwise) | As expected [00:08-0:14] |
| A2. Keeping this position, open the valve. | Valve should open (rotate 180 degrees clockwise) | As expected [00:14-00:18] |
| A3. Placing thumb only on one side of the valve, close the valve. | Valve should close (180 degrees counterclockwise) | As expected [00:18-0:29] |
| A4. Keeping this position, open the valve. | Valve should open (180 degrees clockwise) | As expected [00:29-0:40] |
| A5. Keeping this position again, close the valve looking at the pinching of the tubing from the side. | Should pinch the tube significantly. | As expected [00:40-00:50] |
| A6. Keeping this position open the valve again, looking at the pinching of the tube. | Should release pinching force on the tubing. | As expected [00:50-00:54] |
| A7. Keeping this position, close the valve. | Should pinch the tube significantly and in a way that is consistent with A5. | As expected [00:54-01:08] |
| A8. Rotate the valve in unexpected location (so not a multiple of 90 degrees) and check behaviour. | Valve should remain closed, pinching the tube. | As expected [01:08-1:33] |

Exclusion: The motor and 3D print must be held in place firmly by supporting structure (i.e. if the motor is left to spin freely, it will not close the valve, the entire valve will simply spin).

Constraints

5. Must be gravity fed.

It is indeed gravity-fed and according to the videos from specification 1, it can still provide a sufficient water flow whilst being entirely gravity-fed.

Exclusion: If the valve is not inclined downward (i.e. not in a position that will be conducive to water flow according to Bernoulli’s principle), the water will not circulate.

6. Must be leak-free and remain uncontaminated

The pinch valve allows for a completely leak-free and contamination-free system. This is demonstrated in multiple test videos (i.e. specification 1 and 2).

7. Able to be well integrated into the rest of the system (not too big)

## Test with full system integration. We were able to fully integrate with ease.

## 8. Electrical components should not pose any safety hazards (overheating, contact with water, using the proper resistors)

The system is generally leak-free. Electronics that are placed close to potential water spots are covered (e.g. the wires are covered by tape).

9. Motor and valve must be firmly held in place in the overall prototype structure.

This is a requirement for the overall system discovered in the torque test. Final system integration will confirm this.