March 22nd

- Designed the stairs

  - Decided to laser cut all pieces of the stairs

  - Material: Plywood (Small cost, but durable $4.88), if using acrylic, unsure/unfamiliar with making the joints

  - Decided on the parameters

March 27th

- Decided to have thicker pieces of plywood for the supports (every part except for the top shelf pieces) --> Plywood ¼"

  - Was unsure if such thin plywood would support the weight of all components (tanks filled with water, electronics)

  - Wanted to try laser cutting pieces, so decided to keep the top shelf pieces as laser cut

- Created the sketch for the top shelf pieces (2 x 5”x5” squares) for laser cutting in Fusion360

  - Combined with sketches for parabolic trough pieces in AutoCAD to save material

April 3rd

- Assembled the stairs

  - Cut pieces for the supports of the stairs and the bottom part (dimensions used in document Dimensions of Stairs.png in Structure Design Process folder)

  - Attached all the pieces (cut wooden pieces and laser cut wooden pieces), used wood glue and corner clamps for each joint (refer to Gluing Joints for Stairs.jpg in the Structure Design Process folder)

    - Originally was thinking of using nails+wood glue, however the nails provided by MyFab were too wide, thus decided to only use wood glue

    - Placed glue joints to ensure that there would be minimal shear stress applied on glue joints (weak in shear), made glue joints perpendicular to applied forces

  - Since only able to use 2 corner clamps (one for the top and one for the bottom) and unsure of the time it takes for the wood glue to dry, waited 30 minutes between each joint before removing the corner clamps and doing another joint

April 4th

- Discussed/brainstormed positioning of electronics within the stairs (refer to Potential Positioning on Structure.jpg)

  - Including the breadboards, motors, water level sensor holder, and etc

  - Considering the connection between electronics

  - Prevent contact of water with electronics

- Designed back support to organize the connection between artifacts and devices

  - Decided on the general design

  - Decided to use Plywood ¼" considering the dimensions of support materials needed and durability and inexpensive price of plywood

April 5th

- Created back support by cutting Plywood ¼"

- Outlined the shape of the stairs on back support

- Decided the positioning of other subsystems

  - Discussed how to securely locate the valves within the stairs and back support

April 6th

- Finalized the positioning of subsystems

- Made holes to locate the valves to optimize the physical integration with the tanks

- Modified the stairs to locate the support for the valves

  - To minimize the impact of torque, caused by the rotation of the motors, on the stairs

- Created supports for the valves

  - Made of acrylic considering its durability

April 7th

- Assembled the subsystems, stairs, and back support

- Brackets are used to hold the stairs and back support together

  - Allow disassembly in the future