Integrated Systems Testing Protocol

1. Specification 1: Dispense Water from Boiling Tank to Cold Tank

The integrated prototype should successfully transfer water from boiling tank to cold tank based on the sensor output and corresponding software applications and mechanical control.

This performance of the prototype represents the integration of its software functionality, to detect the water temperature and time to ensure sanitization of water, and mechanical functionality, to control the water flow and transfer between the tanks.

The purpose of this testing is to demonstrate how the overall integrated system interacts and performs. More detailed tests, including edge cases, are conducted during individual subsystem testing.

1.0 Normal Operation

Testing Protocol

- Fill the boiler tank with water, ensuring the water level remains below the entry hole.

- Process Temperature data from the thermistor in boiling tank.

- The valve between the boiler tank and the hot tank is automatically activated based on the temperature data and its corresponding mechanical output, Refer to Table 1, A1 and A2.

- Replace water in hot tank with water at the designated temperature.

- The valve between the hot tank and the cold tank is automatically activated based on the temperature data and its corresponding mechanical output, Refer to Table 2, A3 and A4.

Table 1: Collected Data from Testing Protocol Specification 1

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Fill the boiler tank with its temperature above 40 degrees | Water is dispensed to the hot tank after 40 seconds | As expected [00:45-02:00] |
| A2. Fill the boiler tank with its temperature below 40 degrees | Water is not dispensed | As expected [00:00-00:45] |
| A3. Fill the hot tank with its temperature above 20 degrees | Water is not dispensed | As expected [2:00-02:15] |
| A4. Fill the hot tank with its temperature below 20 degrees | Water is dispensed until the cold water tank’s water level exceeds 4 cm. | As expected [2:15-03:55] |

2. Specification 2: Trough Rotation After System Integration

This specification is part of the final mechanical trough specifications, and is as follows: “The trough’s rotation through all 3 modes (described in Software section) must not interfere with the layout or function of the rest of the system.”

This performance of the prototype represents the capability of the trough and water movement subsystems to operate in parallel without disrupting each other. Note that here, a key difference between the prototype and design concept is that no heating fluid is pumped in the trough tubing, and so the heating tank is not heated by the trough.

Also note that the following test is very similar to Sec 1.0 of the solar trough subsystem testing protocol, which is appropriate as the only difference between pre- and post-integration is the addition of the trough tubing. For the same reason, edge cases are assumed to operate identically to edge-case tests (Sec 1.1 and 1.2 of solar trough subsystem testing protocol).

1.0 Normal Operation

This test is meant to simulate expected operation of the trough. All 3 stages (ie. modes of operation) are shown.

Testing Protocol

Note that # of degrees is defined as increasing clockwise from zero-degree position (trough facing down).

- Have trough post-integration. Have Pico plugged into computer. Have CircuitPython program running.  Have trough at Stage 1 (downward-facing mode). Confirm that trough is at Stage 1 by manually moving trough away from zero position.

- Plug Pico into computer. Load the CircuitPython program to power system on. Observe by referring to Table 1, A1.

- Press the button in the circuit.

- Observe result and compare against expected result. Refer to Table 1, A2.

- Press the button in the circuit.

- Observe result and compare against expected result. Refer to Table 1, A3.

- Press the button in the circuit.

- Observe result and compare against expected result. Refer to Table 1, A4.

- Observe state of integrated system and compare against expected result. Refer to Table 1, A5.

Table 3: Collected Data from Testing Protocol Specification 2

For video of this test, see video titled “Test2_0” in Test Videos folder. Please refer to video timestamps in rightmost column.

| Action | Expected Output | Observed Output |
| --- | --- | --- |
| A1. Trough at Stage 1 | Trough rotates to 0 degrees | As expected [00:00-00:04] |
| A2. Button press | Trough rotates to 270 degrees, then rotates between 90 degrees and 270 degrees repeatedly | As expected [00:04-00:21] |
| A3. Button press | Trough rotates back to 0 degrees, then rotates to 90 degrees | As expected [00:21-00:31] |
| A4. Button press | Trough rotates to 0 degrees | As expected [00:31-00:38] |
| A5. Observe integrated system | Integrated system components are unchanged (have not detached, tangled, etc.) | As expected [00:38-00:47] |