This document addresses the issues the team encountered during the prototyping process and the lessons learned which can be reflected in the future for future engineering design projects.

The lessons are not limited to the product of negative experiences but also positive experiences such as actions or decisions that helped the team throughout the process.

1. Needs of detailed diagrams for assembly and integration

1. 1. Background

The team had difficulties integrating physical prototypes of the valve subsystem with the tank subsystem. The issues encountered include slight misalignment of the motor of the valve with the pre-made hole on the stairs. This aspect of the prototype was significant to prevent potential fracture of the wooden stairs caused by the torque due to the motor's rotation.

1. 2. Team’s Response

The team promptly improvised, modifying the arrangement or dimensions of the elements. This fast response allowed the team to move forward with minimal delay but also sometimes resulted in consequential issues. The modification of the prototype was made after being discussed as a team.

1. 3. Lesson Learned

The team’s system architecture was sufficient to understand the mechanism of the prototype and how different subsystems are connected and communicate with each other. However, the team lacked a detailed diagram for the physical integration of subsystems, such positioning of each component within the entire prototype, or did not thoroughly record the plan after the discussion. Having a detailed diagram and plan for physical integration of subsystems at the beginning of the prototyping process could have minimized the issues the team encountered or the modification needed to deal with the issue.

1. 4. Next Step / Possible Improvements

Having a detailed diagram indicating the positions and dimensions of the elements within the entire integrated prototype will prevent misarrangement and help visualizing the connection between them.

If the team encounters any issues, analyzing the root causes of the issues and potential impact of possible solutions on other aspects of the prototype will prevent additional issues and help keeping track of design modifications.

2. Thorough System Integration Plan involving Anticipation of Possible Issues

2. 1. Background

Each subsystem was working as the team expected, yielding successful testing results. However, the team encountered unexpected technical problems during the process of integration of subsystems, subsystems not communicating with each other properly. For example, the hot-to-cold-tank valve was not turning correctly, which caused the water to not flow correctly in response to opening and closing signals from software.

2. 2. Team’s Response

The team focused on identifying the cause of issues encountered, referencing the edge cases of the subsystems. The team was able to identify and address the issue, but the process was time consuming, delaying the testing process. Troubleshooting was made much more difficult by the fact that after integration, it was less clear what part was causing any particular malfunction (software, electrical, mechanical, which subsystem, which subsystem interaction, etc.). To illustrate, it took us a long time to trace the waterflow problem back to the turning mechanism valve itself, because we had to investigate many possible issues (code bugs, malfunctioning thermistor or water level sensor, etc.).

2. 3. Lesson Learned

The testing and managing the test cases were significantly more difficult for the integrated system compared to each subsystem. Sufficient identification of possible edge cases prior to the integration, including the causes of miscommunication with another subsystem, could have prevented the technical issues during the integration. We could also have made our performance-checking software for the integrated system more robust (ex. with more frequent status update messages) so that we could more readily check software and electrical errors.

Moreover, the testing of the integrated prototype examined the overall functionality of the prototype as the team believe the testing of edge cases were sufficiently done within each subsystem. Having more testing protocols for low level functionalities would have helped the team identify the cause of the problem and gain deeper understanding of the integration and communication of the subsystems.

2. 4. Next Step / Possible Improvements

Anticipation of edge cases prior to integrating will help the team develop more specific testing protocols and structured integration of the subsystems. We could also have tested iteratively as we integrated, instead of just at the end post complete integration, because we would have fewer possible issues to trace and more trust in each component (or component cluster) by the end of the integration process if we did so. This iterative testing could look like: building two systems separately with one valve and tank each and testing them both, adding the valve(s) to the backboard before the tanks and testing just the valve mechanism when attached to the backboard, etc.

3. Agile Project Management including Sprint Planning

3. 1. Background

The agile workflow is the main workflow that our team inspired ourselves from. We used standups in meetings, during our two main sprints. The first sprint was a week before the design review and the second was the week and a half before the final dossier submission. This allowed us to schedule our workflow in accordance with our workload from other subjects and set reasonable work expectations for one another.

3. 2. Team’s Response

Sprints worked well for our team because they helped get everyone on the same page at the right time. We also did a good job at scheduling these well since our workloads aligned this semester. However, they could be stressful when things tended to be done at the last minute. For example, we did not have time to test certain things like the Nano to Pico water level signal at the end of our process.

3. 3. Lesson Learned

In the future, sprints could be scheduled a little more ahead of tight deadlines and we should plan for the unplanned. We learned that things never quite go as planned and although we were all in a focused environment, these challenges were hard to overcome effectively with so little time.

Stand ups helped update everyone on progress and keep each other on the same page but could have been implemented much earlier.

3. 4. Next Step / Possible Improvements

Therefore, sprints could be scheduled and then followed directly by a buffer. On top of this, systems integration will take much longer so we should include some systems integration in previous sprints before trying to tackle it only in the last one. We could also explicitly schedule standups more often because we only started recording them and taking them more seriously near the end of our process.